Technical Data 

GRANT ENGINEERING (UK) LTD  -  OIL BOILER RANGE  -  TECHNICAL DETAILS

| Boiler Name | Boiler Type | Boiler Model | Set output (Btu/h) | Burner Make/Type | Width (mm) | Height (mm) | Depth (mm) | Conv flue Dia (mm) | Connection Sizes Flow Return |  | Exp Vessel Size (litres) | SEDBUK Category | Integral Programmer | Weight Dry (kg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Euroflame | Kitchen/Utility Euroflame Model(top service) | 50/70 | 60,000 | Riello RDB1SH | 345 | 812 | 555 | 100 | 1" BSP | 1" BSP | N/A | C | N/A | 76.0 |
|  |  | 70/90 | 80,000 | Riello RDB1 | 410 | 860 | 600 | 100 | 1" BSP | 1" BSP | N/A | C | N/A | 109.0 |
|  |  | 90/120 | 110,000 | Riello RDB2 | 410 | 860 | 600 | 125 | 1¼" BSP | 1¼" BSP | N/A | C | N/A | 121.0 |
|  | Boiler house | 50/70 | 60,000 | Riello RDB1SH | 345 | 705 | 555 | 100 | 1" BSP | 1" BSP | N/A | C | N/A | 55.0 |
|  |  | 70/90 | 80,000 | Riello RDB1 | 425 | 715 | 620 | 100 | 1" BSP | 1" BSP | N/A | C | N/A | 100.0 |
|  |  | 90/120 | 110,000 | Riello RDB2 | 425 | 715 | 620 | 125 | 1¼" BSP | 1¼" BSP | N/A | C | N/A | 112.0 |
|  | Kitchen System(top service) | 50/90 | 80,000 | Riello RDB1 | 410 | 860 | 600 | 100 | 22mm* | 1" BSP | 12 | C | N/A | 117.0 |
|  |  | 90/120 | 110,000 | Riello RDB2 | 410 | 860 | 600 | 125 | 28mm* | 1¼" BSP | 12 | C | N/A | 129.0 |
| Multipass | Kitchen Multi Pass Model(front service) | 50/70 | 70,000 | Riello RDB1 | 410 | 860 | 605 | 100 | 1" BSP | 1" BSP | N/A | C*** | EPKIT | 115.0 |
|  |  | 70/90 | 80,000 | Riello RDB1 | 410 | 860 | 605 | 100 | 1" BSP | 1" BSP | N/A | C*** | EPKIT | 117.0 |
|  |  | 90/140 | 125,000 | Riello RDB2 | 410 | 1080 | 690 | 125 | 1¼" BSP | 1¼" BSP | N/A | C*** | EPKIT | 148.0 |
|  |  | 140/160 | 160,000 | Sterling 90 | 530 | 1175 | 835 | 125 | 1¼" BSP | 1¼" BSP | N/A | C*** | EPKIT | 210.0 |
|  |  | 160/200 | 200,000 | Sterling 90 | 530 | 1175 | 835 | 150 | 1¼" BSP | 1¼" BSP | N/A | C*** | EPKIT | 210.0 |
|  | Boiler house | 70/90 | 80,000 | Riello RDB1 | 460 | 720 | 625 | 100 | 1" BSP | 1" BSP | N/A | C*** | N/A | 98.0 |
|  |  | 90/140 | 125,000 | Riello RDB2 | 460 | 895 | 655 | 125 | 1¼" BSP | 1¼" BSP | N/A | C*** | N/A | 125.0 |
|  |  | 140/160 | 160,000 | Sterling 90 | 550 | 1000 | 765 | 125 | 1¼" BSP | 1¼" BSP | N/A | C*** | N/A | 197.0 |
|  |  | 160/200 | 200,000 | Sterling 90 | 550 | 1000 | 765 | 150 | 1¼" BSP | 1¼" BSP | N/A | C*** | N/A | 197.0 |
|  | Kitchen System(top service) | 50/70 | 70,000 | Riello RDB1 | 410 | 860 | 605 | 100 | 22mm* | 1" BSP | 12 | C*** | EPKIT | 117.0 |
|  |  | 70/90 | 80,000 | Riello RDB1 | 410 | 860 | 605 | 100 | 22mm* | 1" BSP | 12 | C*** | EPKIT | 117.0 |
|  |  | 90/140 | 125,000 | Riello RDB2 | 410 | 1080 | 690 | 125 | 28mm* | 1¼"BSP | 12 | C*** | EPKIT | 150.0 |
| Combi | Kitchen Combi | 70V3 | 70,000 | Riello RDB1SH | 560 | 860 | 605 | 100 | 22mm* | 22mm* | 12 | C | MTKIT ETKIT or TCRKIT | 158.0 |
|  |  | 90V3 | 90,000 | Riello RDB1 | 560 | 860 | 605 | 100 | 22mm* | 22mm* | 12 | C |  | 158.0 |
|  |  | Max | 113,600 | Riello RDB2.1 | 560 | 860 | 605 | 100 | 22mm* | 22mm* | 12 | C |  | 159.0 |
|  | Outdoor Combi | 90V3 | 90,000 | Riello RDB1 | 617 | 885 | 895 | Green System | 22mm* | 22mm* | 12 | C | TCRKIT | 202.0 |
|  |  | Max | 113,600 | Riello RDB2.1 | 617 | 885 | 895 | Green System | 22mm* | 22mm* | 12 | C | TCRKIT | 203.0 |
| Outdoor | Outdoor Module | 50/70 | 60,000 | Riello RDB1SH | 388 | 829 | 734 | Green System | 1" BSP | 1" BSP | 10 | C | N/A | 94.3 |
|  |  | 70/90 | 80,000 | Riello RDB1 | 505 | 908 | 800 | Green System | 1" BSP | 1" BSP | 10 | C | N/A | 129.7 |
|  |  | 90/120 | 110,000 | Riello RDB2 | 505 | 908 | 800 | Green System | 1¼" BSP | 1¼" BSP | 12 | C | N/A | 138.0 |
|  |  | 110/140 | 140,000 | Riello RDB2 | 466 | 1045 | 837 | Green System | 1¼" BSP | 1¼" BSP | 12 | C | N/A | 167.0 |
| Vortex | Kitchen | 15/26 | 78,475 | Riello RDB1 | 470 | 860 | 603 | 100 | 22mm* | 22mm** | N/A | A | EPKIT | 119.0 |
|  | Kitchen System | 15/26 | 78,474 | Riello RDB1 | 470 | 860 | 603 | 100 | 22mm* | 22mm** | 10 | A | EPKIT | 127.0 |
|  | Utility | 15/26 | 78,475 | Riello RDB1 | 470 | 860 | 603 | 100 | 22mm* | 22mm** | N/A | A | N/A | 112.0 |
|  |  | 26/36 | 107,500 | Riello RDB2 | 470 | 900 | 603 | 100 | 28mm* | 28mm** | N/A | A | N/A | 134.0 |
|  |  | 36/46 | 142,000 | Riello RDB2.1 | 470 | 900 | 603 | 100 | 28mm* | 28mm** | N/A | A | N/A | 134.0 |
|  | Utility System(top service) | 15/26 | 78,475 | Riello RDB1 | 470 | 860 | 603 | 100 | 22mm* | 22mm** | 10 | A | N/A | 126.5 |
|  |  | 26/36 | 107,500 | Riello RDB2 | 470 | 900 | 603 | 100 | 28mm* | 28mm** | 12 | A | N/A | 142.0 |
|  |  | 36/46 | 142,000 | Riello RDB2.1 | 470 | 900 | 603 | 100 | 28mm* | 28mm** | 16 | A | N/A | 142.0 |
|  | Outdoor Module | 15/26 | 78,475 | Riello RDB1 | 505 | 908 | 800 | Green System | 22mm* | 22mm** | 12 | A | N/A | 157.0 |
|  |  | 26/36 | 107,500 | Riello RDB2 | 505 | 908 | 800 | Green System | 28mm* | 28mm** | 16 | A | N/A | 174.0 |
|  |  | 36/46 | 142,000 | Riello RDB2.1 | 505 | 908 | 800 | Green System | 28mm* | 28mm** | 16 | A | N/A | 174.0 |
| Vortex Combi | Kitchen Combi | 26 | 88,700 | Riello RDB1 | 590 | 860 | 605 | 100 | 22mm* | 22mm* | 12 | A | MTKIT/ETKIT | 188.0 |
|  |  | 36 | 123,700 | Riello RDB2.1 | 615 | 900 | 605 | 100 | 28mm* | 28mm* | 14 | A | MTKIT/ETKIT | 218.0 |

*Push-fit connectors supplied with boiler 
**Compression fittings required (not supplied with boiler) 
***SEDBUK efficiency capped at 85.7% for category 'C'